GETCLK Version 0.1  [For IBM-Compatibles]

CP/M-86 System Time/Date Setter For PCs and XTs (Y2K Compliant)

Freeware from Kirk Lawrence

GETCLK sets the CP/M-86 system clock to the date and time contained in

the battery-operated PC/XT hardware clock.  This program is Y2K-compliant

(see note, below).

Simply invoke GETCLK from the command line (or from an AUTOEXEC.SUB-type

of start-up file).  GETCLK will automatically set the CP/M-86 system

clock and calendar, and update the information on CP/M-86's status line

to reflect the settings retrieved from the hardware clock.

Naturally the hardware clock, itself, must be set correctly in order for

the system date and time to be accurate! :)

GETCLK is designed to work with the generic add-in clocks that are commonly

found in 8086/8088-based PC- and XT-class computers.  These clocks use

standardized hardware port addresses of 240h or 340h...and this is where

GETCLK looks for the date and time information.

If your computer's clock is located at a non-standard hardware address,

or if you have a proprietary clock that needs specialized clock-setting

software (such as the Dallas Semiconductor "Smartwatch," for example),

then GETCLK probably won't work for you.

If you aren't sure about your particular hardware, go ahead and run GETCLK

anyway.  If the clock in your computer isn't compatible, GETCLK will exit

with an error message, and no harm will be done.

=> NOTE ON Y2K COMPLIANCE: Most generic PC/XT hardware clocks do not keep

track of the century.  This is good, because it means that the clock can

theoretically be used forever.  On the other hand, it can also be a

problem...because there's no way for the hardware clock to let CP/M-86

know that the century has changed.

Therefore, in order to be Y2K compliant, GETCLK must make an arbitrary

assumption about the current century.  Here's the Y2K logic that GETCLK

uses: it assumes that we're in CP/M-86's default century (normally 19xx)

if the "year" in the hardware clock is between 80 and 99.  But if the

"year" in the hardware clock is between 00 and 79, then GETCLK assumes we

need to advance CP/M-86's default century to the =next= century (normally

20xx).

GETCLK's setting of the "century" in the CP/M-86 system clock is done

according to this assumption.

In normal use, this logic works great.  Occasionally, though, a situation

can arise where the assumption is not valid.  For those cases, GETCLK

allows you to =MANUALLY OVERRIDE= its Y2K logic.

To force GETCLK to =not= change CP/M-86's default century, use a forward-

slash ( / ) as a command line argument when you invoke the program:

GETCLK /

When would you need to do that?  Usually, only in these two instances:

1. If you're running a copy of CP/M-86 that has been "patched" for 21st

century compatibility, then you MUST use the "/" switch.

(A better solution for users of a "patched" version of CP/M-86 would

be to use the non-Y2K-compliant clock-setting program GETCLOCK.CMD.)

2. If you invoke GETCLK more than once during the same CP/M-86 session

without rebooting the computer, then you MUST use the "/" switch.

The reason why the "/" switch is necessary in these cases is quite technical.

It has to do with the low-level internal mechanisms required by CP/M-86 in

order to manipulate the "century" setting.  Since technical explanations are

beyond the scope of this documentation, suffice it to say that if the override

switch is not used in the examples outlined above, CP/M-86's "century" setting

will become messed up.

But in normal, ordinary, everyday use -- where you'll run GETCLK only once

in a given session -- the "/" override switch should never be necessary.

It's only there as an accommodation for those few strange and unusual

situations.

GETCLK does not work with AT-class computers (286 and higher).  Use the

program GETATCLK.CMD if you have an AT-class machine.

GETCLK.CMD is free.  As is always the case with "freeware," there are no

guarantees whatsoever.  If you use the program, you do so entirely at your

own risk and peril.  Your use of GETCLK.CMD shall be prima facie evidence

under law that you understand and accept these terms and conditions.

============================================================================

Other CP/M-86 Clock-Related Utilities By The Same Author:

GETCLOCK - Non-Y2K-compliant system time/date setter for 8088 PCs and XTs.

GETATCLK - System time/date setter for AT-class (286 and up) IBM clones.

SETCLOCK - Sets the PC/XT hardware clock to the CP/M-86 system time/date.

NEWTOD   - Manual Y2K-compliant time/date/century setter; replaces TOD.CMD.

CENTURY  - Advances the "century" setting of CP/M-86's clock/calendar.

CLK      - Puts a ticking, digital "clock" on the screen.

#