NetWare Workstation for OS/2 v2.01

Readme file

Remote Workstations Not Supported

---------------------------------

The files needed to install and use remote workstations are

not included in this release. The RIPL files will be available

at a later date.

Frame Type Default Is Different

-------------------------------

The default frame type for Ethernet ODI drivers (such as

NE2000.SYS) has changed. In NetWare v3.x and NetWare v2.x,

Ethernet drivers defaulted to Ethernet_802.3. In NetWare v4.0,

they default to Ethernet_802.2.

NetWare v4.0 server drivers for Ethernet also default to

Ethernet_802.2. Servers and workstations use the same frame

type to communicate with each other.

Routers on the network also have to support the frame type or

your workstation often will not be able to get a connection.

Some routers currently in use on your network may not support

the new Ethernet_802.2 default.

Your workstation may not be able to connect to a network

expecting Ethernet_802.3 if you use the Ethernet_802.2 default.

To eliminate a potential conflict, you can define both frame

types (Ethernet_802.2 and Ethernet_802.3) on your network. For

the workstation, define frame types in the NET.CFG file.

Include a line similar to the following, replacing NE2000 with

the name of your ODI driver:

link driver ne2000

frame ethernet_802.2

frame ethernet_802.3

The first frame defined is the only one used for the initial

Get Nearest Server request. Therefore, if you have some

servers that are using only one frame type, such as

Ethernet_802.3, put that frame type first. That way your

workstation will be able to make a default connection to those

servers.

See Chapter 5, "Network Boards and Drivers" of NetWare

Workstation for OS/2 for more information about frame type.

Logging In From Global DOS Not Supported

----------------------------------------

Do not log in from a global DOS session. Instead, log in from

an OS/2 session and then switch to the DOS session.

Except for this login difference, you can still set up drive

mappings for global sessions as instructed in Chapter 3,

"Network Access From Virtual DOS and MS Windows" of NetWare

Workstation for OS/2.

Editing Login Scripts for DOS Sessions

--------------------------------------

If a user logs in from a private DOS session running NETX.EXE,

a NetWare v3.11 user login script will execute if it exists.

This login script is called LOGIN and is located in the

SYS:MAIL\userID directory. You can edit it with a text editor

or a bindery utility from previous versions of NetWare such as

SYSCON.

If the server is a new NetWare v4.0 installation rather than

an upgrade, login scripts and the \MAIL\userID directory will

not exist. Login scripts in NetWare 4.0 are properties of the

user object.

You can create the login script on a local drive and use the

/S option when you login. For example, create the LOGIN.DOS

login script on the C:\ drive and type the following to log in:

LOGIN /S C:\LOGIN.DOS

Running NetWare v4.0 Utilities in DOS_STARTUP_DRIVE Sessions

------------------------------------------------------------

In a private DOS session booted from a real DOS kernel

(i.e., a session with DOS_STARTUP_DRIVE set) running NetWare

v4.0 DOS Requester, you may receive errors running some of the

utilities.

Solve the problem by being logged in from an OS/2 session

whenever you use a utility from the DOS_STARTUP_DRIVE session.

You must be logged in to the same server from the OS/2 session

as you are logged in to from the DOS_STARTUP_DRIVE session.

For example, suppose you logged into server SALES from the

DOS_STARTUP_DRIVE session. For some utilities to function, you

would also need to log in to server SALES from an OS/2

session. As long as your OS/2 session is logged in, your

utilities will function properly.

This condition particularly applies to the PCONSOLE, NETUSER,

FILER, NDIR, and NPRINT utilities.

Request Retries Default Incorrect

---------------------------------

The Workstation for OS/2 manual and online help incorrectly

document the default for the "NetWare Requester request

retries" NET.CFG setting. The correct default is 10, not 20.

This lower default helps minimize transmission errors with

slower ODI drivers.