Installing and Setting the IBM Token-Ring 16/4 Busmaster Server Adapter/A

The IBM Token-Ring 16/4 Busmaster Server Adapter/A is functionally different

from the IBM Token-Ring 16/4 Adapter/A in two ways:

�    Busmaster capability

Busmastering allows the IBM Token-Ring Network 16/4 Busmaster Server Adapter/A

to transfer between the network and the server's memory without the aid of the

server's CPU.  The server's CPU can then process other network tasks more

quickly than it otherwise could.

�    Downloadable microcode

Downloading microcode allows you to upgrade the IBM Token-Ring Network 16/4

Busmaster Server Adapter/A's firmware without replacing any chips on the

adapter.  FIRMLOAD.COM (supplied by Novell) allows you to download the

firmware (supplied by IBM) to the adapter.

Before you install this adapter in a 3.1x server, you should have designed

your network and assembled the necessary hardware.  If you have any questions

about Token-Ring hardware or network layout, see the Novell IBM Token-Ring

Network Supplement manual for further information.

For information concerning installation and configuration of the 16/4

Busmaster card, please refer to the IBM Token-Ring Network 16/4 Busmaster

Server Adapter/A Installation and Testing Instructions that accompany the

adapter.  Additional information concerning the following setable parameters

can be found in the "change configuration" selection in the Reference program:

�    Data rate

The default data rate for the adapter is 4 Mbps. The other valid data rate is

16 Mbps.

Important:  You must set all IBM Token-Ring 16/4 adapters on a ring to use the

same data rate (either 16 Mbps or 4 Mbps).  Failure to do so will cause

network errors.

�    Interrupt level

The default is Interrupt 2.

Other valid levels include interrupts 3, 4, 5, 7, 10, 11, and 12.

�    I/O address

The default I/O address is 86A0-86AF.

Each valid address consists of a 16-byte block of RAM within the Micro Channel

machine.  There are seven additional valid I/O addresses:  96A0, A6A0, B6A0,

C6A0, D6A0, E6A0, and F6A0.

Ensure each adapter installed in the machine has a unique I/O address.

�    Arbitration level

The default arbitration level is E.

Other valid arbitration levels are 0, 1, 2, 3, 5, 6, 7, 8, 9, A, B, C, and D.

Each busmaster adapter requires a unique arbitration level for negotiating

with other busmaster adapters within the same machine for control of the

microchannel bus.

�    Fairness

The default is to operate with fairness ON.

The other valid option is to operate with fairness OFF.

Fairness determines how a Micro Channel busmaster adapter negotiates with

other busmaster adapters within the same machine for control of the

microchannel bus.

For more information about arbitration and fairness, read the IBM publication,

PS/2 Hardware Technical Reference.

�    Bus release

The default is "Maximum."

The other valid setting is "Immediately."

The bus release setting determines how long a Micro Channel busmaster adapter

has to relinquish the bus when another adapter preempts it during a DMA

operation.

A setting of "Immediately" causes the adapter to relinquish control of the

Micro Channel bus upon receiving the preempt signal.

A setting of "Maximum" allows the adapter time to finish critical DMA

operations, resulting in better overall performance.

�    Early Token Release (ETR)

When the data rate is 16 Mbps, the default is to operate with ETR.

The other valid option is NO ETR.

Operation with ETR is not valid for Token-Rings with a data rate of 4 Mbps.

ETR can increase network performance by causing the adapter to release the

token immediately upon deencapsulating the data packet, rather than after it

has successfully transferred the deencapsulated data to the next higher

network protocol.

DOWNLOADING the Microcode

IMPORTANT

The Microcode must be downloaded to the Busmaster Server Adapter/A each time

the machine is powered up.  This can be accomplished manually (from the

command line) or can be automated in a batch file.

The IBM Token-Ring 16/4 Busmaster Server Adapter/A requires microcode that is

not resident on the adapter.  The microcode is contained in two files included

on the adapter's option diskette (supplied by IBM):

�    MONT400.BIN

�    WRTRAM.BIN

To download MONT400.BIN and WRTRAM.BIN to the adapter, you must use

FIRMLOAD.COM, a utility supplied by Novell.

When you have located these three files (including FIRMLOAD.COM), complete the

following steps:

1.   Copy the MONT400.BIN, WRTRAM.BIN, and FIRMLOAD.COM files to the same

diskette (or to the same directory).

2.   Change to drive or directory that contains MONT400.BIN, WRTRAM.BIN, and

FIRMLOAD.COM.

3.   Type the following:

FIRMLOAD  <Enter>

A message similar to the following appears:

MESSAGE

Novell Token-Ring DownLoad Utility for IBM DMA Bus Master

v1.00 (901129)(C) Copyright 1990 Novell Inc.  All Rights

Reserved.

IBM Token-Ring DMA Bus Master FOUND in Slot 2;

Data Rate = 04 Mbps

DMA Level = 0E;  Interrupt Level=02

Node Address = 1000 5A4F 00AE;    Port = 86A0

IBM Token-Ring DMA Bus Master DownLoad SUCCESSFUL

IMPORTANT

Applies to NetWare 3.11 only

The loader portion of SERVER.EXE will need to be modified to properly load the

new TOKENDMA.LAN driver.  To make this change in SERVER.EXE, you need to use

XLOAD.EXE and LDR001.PTF.  Novell recommends patching SERVER.EXE on NetWare

3.11 servers except NetWare  SFT III 3.11.  NetWare 3.12 does not require the

new loader because it is built into SERVER.EXE.

For more information about XLOAD and LDR001, read XLOAD.DOC and LDR001.DOC.

XLOAD should always be applied when using interrupts 2 and 9 on Micro Channel

bus machines.

-----------------------------------------------------------------

Any trademarks referenced in this document are the property of their

respective owners.  Consult your product manuals for complete trademark

information.

-----------------------------------------------------------------