HQ4/GAMERA IDE BRINGUP DOCUMENT

===============================

/*

*

*

*  $Revision: 1.2 $

*/

Where is the latest IDE ?

bootp()recycle:shell.ide

What are the commands available in IDE ?

After you boot up the latest IDE...

Type all   - this displays all the commands supported in IDE

Type works - this displays all the commands that are working

The rest of the document lists all the commands we are planning to

bringup in the graphics lab.

command:-	mg_link_chk

Usage:-		mg_link_chk <port number>

Description:-   This command reads the XBOW register to check if a widget is

present in <port>

mgras_xbow_link_check (mgras_xbow.c)

Status:-

command:-	mg_setboard

Usage:-		mg_setboard <port>

Description:-	This command probes the presence of Gamera in the specified

xtalk port and displays if it is found it or not

If the Gamera board is found, the command resets the board.

mgras_setboard (mgras_info.c)

mgras_hq4probe (mgras_xbow.c)

badaddr

_mgras_islinkalive (mgras_xbow.c)

_mgras_hq4linkreset (mgras_xbow.c)

Status:-

command:-	mg_hq4poke

Usage:-		mg_hq4poke -o <reg offset> -v <value>

Status:-

command:-	mg_hq4peek

Usage:-		mg_hq4peek -o <reg offset>

Status:-

command:-	mg_hq4reg

Description:-	walks ones/zeros on some of the HQ4 registers

mgras_hq4_reg_test (mgras_xbowlink.c)

_mgras_hq4_reg_test32 (mgras_xbowlink.c)

Status:-

command:-	mg_hqfifoinit

Description:- 	initializes c/b/d fifo low/high watermarks and delays

mgras_hq_fifo_init (mgras_info.c)

Status:-

command:-	mg_gfxinfo

Description:- 	displays the revs and h/w inventory of various ASICs and DCB

devices

mgras_gfx_info

Status:-

command:-	mg_hq3

Description:-	This tests all of the functions of the HQ4

mgras_hq3_test (mgras_hq3.c)

_mg_set_ZeroGE (mgras_info.c)

mgras_HQGIObus (mgras_hq3.c)

mgras_HQbus (mgras_hq3.c)

mgras_HQ3dctctrl_test (mgras_hq3.c)

mgras_tlb_test (mgras_hq3.c) ??????

mgras_hq3_reif_reg_test (mgras_hq3.c)

mgras_hq3_reif_wdReg (mgras_hq3.c)

mgras_wrHQ3word

mgras_hq3_hag_reg_test (mgras_hq3.c)

mgras_hq3_hag_wdReg (mgras_hq3.c)

mgras_ucode_test (mgras_hq3.c)

mgras_hq3ucode_DataBus (mgras_hq3.c)

mgras_hq3ucode_AddrBus (mgras_hq3.c)

mgras_hq3ucode_AddrUniq (mgras_hq3.c)

mgras_hq3ucode_mem_pat (mgras_hq3.c)

mgras_hq3ucode_walking_bit (mgras_hq3.c)

Status:-

command:-	mg_cfg_wr

Usage:-		mg_cfg_wr -v[config value]

Description:- 	writes the config register with the value given

mgras_brdcfg_write (mgras_info.c)

Status:-

command:-	mg_pcd_wr

Usage:-		mg_pcd_wr byte0 byte1 byte2 byte3 byte4 byte5 byte6 byte7

Description:- 	programs the pixel clock doubler

mgras_pixel_clock_doubler_write (mgras_info.c)

command:-	mg_vc3addrsbus, mg_vc3databus

Description:-	VC3 Bus tests

mg_vc3addrsbus, mg_vc3databus (mgras_vc3.c)

Status:-

command:-	mg_dacreset

Description:-	Resets the DAC

mg_dacreset (mgras_dac.c)

Status:-

command:-	mg_dacpll

Usage:-		mg_dacpll -r[R value] -v[V value] -c[CTRL value]

Description:-	Sets the PLL values of the DAC

mgras_DacPLLInit (mgras_dac.c)

Status:-

command:-	mg_vc3init

Usage:-		mg_vc3init -t 0x107

Descriton:-	load timing table into VC3 and init DAC PLL

mg_vc3init (mgras_vc3.c)

command:-	mg_cmaprev, mg_cmapaddrsbus, mg_cmapdatabus

Description:-	CMAP DCB commands

mg_cmaprev, mg_cmapaddrsbus, mg_cmapdatabus (mgras_cmap.c)

Status:-

command:-	mg_reset

Description:- 	resets the entire graphics hardware

First loas the timing table by using mg_vc3init -t 0x107;

Then run mg_reset -s

Status:-

command:-	mg_rtpio_loop

Usage:-		mg_rtpio_loop -i <in_format> -o <out_format>

Description:- 	PIO-writes values to the Texture fifo and reads the values

from the video fifo.  This uses the diag mode.

mgras_rtpio_loop (mgras_rtdma.c)

Status:-

command:-	mg_rtdma_loop

Usage:-		mg_rtdma_loop -p<#pages>

Description:-   DMA-writes to the Texture fifo and DMA-reads the values

from the video fifo.  This uses the diag mode.

mgras_rtdma_loop (mgras_rtdma.c)

Status:-

command:-	mg_dcbdma

Usage:-		mg_dcbdma <on/off>

Description:- 	Turns on (1) or off (0) the DCB DMA flag

Status:-