Colographic Communications Corporation

Pro Lightning V+ PCI 612168 and 612169

How to Multi-Screen Windows

1) Remove any display adapters currently installed or disable the display

subsystem on your computer's Motherboard.

2) Install Colorgraphic Card(s).

3) Install the Device Driver on this diskette.

Card Installation

Each card installed must have a unique adapter ID assigned via dip

switches 3 and 4 on SW1 to permit system software to activate the cards

independently.

SW1  (As shipped)

����������������������Ŀ ����������������������Ŀ ����������������������Ŀ

� ڿڿڿڿ             � � ڿڿ  ڿ             � � ڿڿڿ               �

� ��������ڿڿڿڿڿڿ � � ����ڿ��ڿڿڿڿڿڿ � � ������ڿڿڿڿڿڿڿ �

�         ������������ � �     ��  ������������ � �       �������������� �

������������������������ ������������������������ ������������������������

1st Card		       2nd Card 		3rd Card       etc...

Switches 1 and 2 select the port addresses used by software to select the

card and each section of the selected card.  The port addresses should only be

changed if there is an I/O address conflict with another card in the system.

Switch	  1   2

328H 32CH -- As Shipped  ON  ON

338H 33CH		 ON  OFF

348H 34CH                OFF ON

358H 35CH		 OFF OFF

Device Driver Installation

1) Exit Windows

2) Run SETUP.EXE from your Windows directory.

3) Select "Display:" and press Enter.

4) Scroll to the end of "display type" list to "Other" and press Enter.

5) Specify A:\W31 as the path on the diskette.

6) Install this diskette in the specified drive and press Enter.

7) Select the desired font size from the displayed list and press Enter.

8) Press Enter to Accept the Changes.

9> If small fonts is selected the default resolution will be 640x480

256 colors, with LARGE fonts selected 1024x768 256 colors will be the

default resolution.  The resolution, colors or refresh rate can be

changed once in Windows in the Control Panel by double-clicking on

SetArray.  Setarray will provides a dialog box to change various

settings of the display driver.

The drivers will automatically reconfigure for more displays as cards

are added to the system or a specific configuration can be designated

via the "system.ini" file.  After running Setup the following section

will be added to the "system.ini" file.  This is a partial list of the

parameters that are added to the [DisplayArray] group.	You may change

the driver operation by changing these parameters using a text editor

of your choice.

[DisplayArray]

screen-size=1280	 -- Resolution	640/800/1024/1152/1280/1600

color-format=8		 -- Color Depth  8=256, 15=32K, 16=64K, 32=16.7M colors

V=1			 -- Number of displays vertically.  (rows)

H=2			 -- Number of displays horizontally.

ClearOnExit=1		 -- Clear all displays when exiting from Windows

RectDisable=1		 -- Windows 3.1 has an acknowledged bug with drivers

that support rectangles.

dpi=96			 -- dpi=96 for small fonts  dpi=120 for large fonts

Hz=60			 -- Vertical Refresh Rate (see chart below)

��������������������������������������������������Ŀ

Resolution Selected �640x480    �800x600    �1024x768	�1280x1024    �

��������������������������������������������������Ĵ

Available Rates �60/72/75   �56/60/72/75�43/60/70/75 �45/60/72/75  �

����������������������������������������������������

Note: Only 49Hz is available at 1600x1200 or 1152x864.

WARNING!! Your Monitor must be capable of the refresh rate at the designated

resolution that you select or damage may occur!!  If you are

uncertain of your monitor's capabilities try the lowest selectable

refresh rate.

If either the Horizontal or Vertical count = 0, the driver will

auto-configure for the number of adapters installed.

The dpi variable is only applicable for resolutions greater than 640 x 480.